# POLICY & PROCEDURE
HAZARDOUS AND MEDICAL WASTE

## Policy Statement

All hazardous and medical waste will be managed according to federal, state and local rules and regulations. There will be a designated person who is knowledgeable of federal, state and local rules and regulations governing the management of any hazardous and/or medical waste for which the facility may be responsible.

## Definitions

### Hazardous Waste:

- Corrosive - a chemical capable of dissolving or wearing away of substances, including tissue, such as glutaraldehyde

- Flammable - easily catches fire and tends to burn rapidly, such as formaldehyde vapors

- Reactive - a reactive chemical is capable of participating in a rapid and sometimes violent reaction with other chemicals and substances, such as mixing phenol and formaldehyde

- Toxic - can cause injury or death if swallowed, inhaled, or absorbed through the skin, such as anti-neoplastic drugs or mercury

### Medical Waste:

- Any waste used in a health care facility that, if not contained and managed properly, could result in the transfer of infection, such as:

  - microbiological wastes

  - blood and blood products

  - pathological  wastes

  - used sharps

  - animal wastes

  - communicable disease, CDC class 4, isolation wastes

  - contaminated lab wastes

  - surgical and autopsy wastes that were in contact with infectious wastes

  - dialysis wastes

  - contaminated medical equipment

  - unused discarded sharps

### Nuclear Waste:

- Any substance that is radioactive

## Procedure

### Identify Waste

Determine by definition if the waste generated will be “Hazardous”, “Medical” or “Nuclear” Waste?

### Labeling

  - Hazardous Waste - label as follows:

    - “hazardous waste” in large print

    - description of contents

    - date collection in the container began

    - name, address, business phone number, state permit or identification number (if applicable) for the generator of the waste

    - name, address, business phone number, state permit or identification number (if applicable) of the transporters, treatment facilities, or other persons to whose control the medical waste is transferred

    - date upon which the hazardous waste was packaged

    - if possible attach MSDS label

  - Medical Waste - label as follows:

    - “Biomedical Waste” or “Medical Waste” in large print

    - international biohazard symbol if possible

    - description of contents

    - date collection in the container began

    - name, address, business phone number, state permit or identification number (if applicable) for the generator of the waste

    - name, address, business phone number, state permit or identification number (if applicable) of the transporters, treatment facilities, or other persons to whose control the medical waste is transferred

    - date upon which the medical waste was packaged

  - Treated waste - label as follows:

    - name, address, business phone number, for the generator of the waste

    - date when the waste was treated

    - treatment method utilized

    - statement indicating that the waste has been treated and is no longer medical waste

  - Write labels using permanent ink and secure label in a manner that will prevent unintentional removal.

  - Use lettering no less than one inch in height and ensure the wording is readily visible if the container is in a lateral position.

### Storage

  - Segregation and Containment

Segregate all waste materials at the time they are produced to prevent:

    - cross contamination of non-hazardous or medical waste

    - dangerous reactions with other wastes or materials

    - maintain all hazardous and medical waste in an appropriate container per federal, state and/or local regulations

    - frequently clean and/or disinfect carts used to transfer waste materials within the medical practice and do not use them for other duties

    - store containers in the area where the waste is generated, if feasible

  - Hazardous Waste

    - determine the maximum amount of waste that will be generated monthly

    - contact the EPA and the RCRA to determine:

      - if a permit is needed to store any of the hazardous waste generated

      - the appropriate container type for any of the hazardous waste generated

      - the total allowable monthly volume that can be stored

      - the environmental conditions that may be required for storage including security

      - recommended inspection parameters and schedule

    - keep all containers secure and closed unless being added to or emptied

    - manage and store cytotoxic (anti-neoplastic agent) waste per OSHA guidelines

  - Medical Waste

    - store medical waste near the area of generation or at the area of pick up if it will be transported off-site

    - secure sharps containers from public access and consider full at ¾ of total volume

    - maintain storage areas used for medical waste:

      - ensure they are durable, easily cleaned, and impermeable to liquids, protected from vermin and other vectors, and have proper environmental conditions and ventilation

      - do not use carpet or flooring with seams

      - secure and restrict access to designated personnel

      - post with biohazard symbols and warning signs both outside and inside

### Disposal

  - Determine what the most effective method of disposal for this organization is based upon, but not limited to:

    - federal, state and local regulations governing the type of waste generated

    - resources available to the organization:

      - financial

      - facility size

      - personnel

      - liability

  - On-site Disposal

    - follow all federal, state and local regulations and guidelines

    - maintain any required permits

    - maintain records per OSHA and RCRA regulations and guidelines

  - Off-site Disposal

    - determine an appropriate contractor based upon, but not limited to:

      - services offered

      - environmental offices referral

      - liability coverage

      - contingency strategies and capabilities

      - service record

      - interview

      - cost

      - contract

    - maintain up to date regulations and confirm the contractor is following them

    - maintain records per OSHA and RCRA regulations and guidelines

### Contingency Planning

The contingency plan will provide reasonable means for back up storage and transportation should the primary system(s) fail.

  - have at least one extra container for each type of waste if feasible

  - have access to at least one alternative transportation system for both medical and hazardous waste

  - have a back up power source if power is needed to maintain waste storage

### Spills

  - Small Spills

    - train designated personnel with regards to management of small spills

    - maintain an appropriate “Spill Kit” within the vicinity of waste generation for the type of waste being handled

    - hazardous waste spill kit should contain at least:

      - several bags of kitty litter, saw dust or sand to absorb spilled liquids

      - brooms, dustpans and a square mouth shovel to sweep up the absorbent material

      - absorbent pillows or booms (available from spill kit suppliers) to contain larger liquid spills and prevent spills from entering drains

      - heavy duty plastic bags or plastic drums (with a lid) to contain hazardous  material prior to disposal

      - appropriate personal protective clothing (such as chemical resistant gloves, safety glasses)

      - a wheelie bin to contain all the above equipment and store hazardous material prior to disposal

    - medical waste spill kit should contain at least:

      - a long sleeve gown, latex gloves, face mask with eye shield and shoe covers

      - disinfectant spray solution

      - towels for cleanup

      - Pick Up Powder with spatula

      - biohazard bags

  - Large spill

Refer to P&P: Emergency Response.

### Nuclear Waste

Be familiar with and follow all Nuclear Regulatory Commission standards and regulations pertaining to any nuclear agents used on-site.